Haas Inspection post – Calibration Data and
Commissioning Mode

Haas Inspection Post Processor

The Haas post will need calibration macro numbers filled in before use. These values are entered in the Post properties fields when post processing from Fusion. Below is a brief explanation of the properties which should be set or confirmed.

There are 3 macro variables that define calibration data. Using Renishaw as an example, look for these 3 values, normally around macro variable #556.

- The calibrated radius will be just less than the actual radius (Renishaw default #556 and #557)

- The eccentricity (run-out in X) will be close to 0 (default #558)

- The eccentricity (run-out in Y) will also be close to 0 (default #559)

If the controller does not have values in macro variable #556, check for values as described above: two values just below the calibrated probe radius and two values very close to 0. In the Haas Next Generation controller macro variables #500 and #10500 are equivalent. If macro variables #556, #557, #558, and #559 are all 0 then you should look for the variable values matching the described pattern, or else contact the Renishaw team to know the exact variables.

A screenshot of these items highlighted in Fusion's post properties is shown in the Post Processor Settings section with some sample values. Once they are filled in with the correct macro variable values and commands, commissioning mode can be set to false, and then inspection programs can be posted.

To double check if you are looking at the proper variables, run the probe calibration process and see if the values are slightly changed.

Post Processor Settings

Here are some of the more common post-processor settings that you will need to pay attention to when post-processing your Inspection toolpath

- Commissioning Mode: Enables M0 and messages at key points in the program. You can disable Commissioning Mode after a successful run.

- Probe calibration Method: The probe tip radius can be calibrated as a single value or two values (in X&Y direction).
If Renishaw is selected as a calibration method, then the probe tip radius will be calculated as an average value of two numbers. Calibrated Radius #500 variable will be taken as the first value and the next variable #501 will be taken as the second.

- Tool offset type: Depends on your tool table type:

  - Memory A – single column to store geometry and wear. You should choose Geometry only

(GEOMETRY ONLY)

#8=#[2000 + #4111]

  - Memory B or C – two columns or more. You should choose Geometry and Wear
(GEOMETRY AND WEAR)
#8=#[2000 + #4111] + #[2200 + #4111]

- Probe On/Off Command: It can be an M-code or a subprogram call. Check that the probe flashes green after the probe on command has been executed.

Renishaw Inspection+ cycles probe On and probe Off command will typically be G65 P9832 and G65 P9833. Check with the Machine documentation if needed.

Commissioning Mode

Inspection posts for Fusion defaults to Commissioning Mode. When Commissioning Mode is enabled, it will print messages on the controller screen at certain places throughout the inspection process for safety purpose.

The following checks are common while running in commissioning mode.

- Ensure the probe is enabled.

  - The probe should move to the first point.

  - Verify the probe is enabled, and if so, press Cycle Start to continue.

- Probe is about to contact part. Move should stop on contact.

  - Probe will position to the measuring point.

  - Make sure the probe is at the correct location and is about to make contact with the surface.

  - If it is positioned correctly press Cycle Start to continue.

  - Probe will touch the measuring surface and retract to a safe point.

- Ensure the probe has been disabled.

  - The probe should move to a safe Z position.

  - Make sure the probe is disabled and press Cycle Start to continue.

- The Results file can now be found in the same location as your NC code file or the folder specified in the Results file location property.

  - The Results file will be saved in the specified file folder.

  - Press Cycle Start to continue.

  - The Z-axis will now move to the machine's maximum Z location.

It is always recommended to enable Commissioning Mode the first time you run an inspection program. 
After the first run, if everything works as expected, Commissioning Mode can be disabled when post processing.

Tutorial:

Follow the below link to see how to use surface inspection with a Haas Next Generation Control. https://www.youtube.com/watch?v=d9VnOJoQbvg

Settings for Multi-axis Probing

For 3+2 probing operation the machine kinematics needs to be set on the post processor. While post processing, the Machine model or Rotary axes can be specified in the post property dialog as shown below.

Or else the corresponding machine configuration file can be selected while post processing.

Refer to the following forum link for instructions on how to set the post as multi-axis.

How to set up a 4/5 axis machine configuration

Reach out to us via HSM Post Processor Forum - Autodesk Community if you have any issues.